# N6Cam Phase 1 — Integration Document

Vendor: Kamacode Ltd. Client: ITP / Novex Target: SIANA N6Cam Edge AI Sensing Kit (STM32N657 + ATON NPU) Date: 2026-05-23 Status: M1 + M2 delivered (v1.4.x), M3 firmware-side cellular plane ready (v1.5.x), awaiting MangOH hardware for end-to-end M3 integration; M5 acceptance + soak procedure documented below.

## 1. Hardware setup

USB-C (CDC shell + UART trace + power)
                       │
            ┌──────────┴───────────┐
            │   Dev host (laptop)  │
            └──────────┬───────────┘
                       │
              ┌────────┴────────┐    USART2 J503 pins 7/9/11/13
              │  SIANA N6Cam    │────────────────────────────────►  MangOH Yellow + WP76 modem
              │  STM32N657      │    115200 8N1 HDLC               (M3/M4 — hardware pending)
              │  ATON NPU       │
              │  IMX335 camera  │
              │  SD card slot   │
              └─────────────────┘

### 1.1 Pin map — N6Cam side (J503 header)

| Signal | J503 pin | STM32N657 pin | Function | Direction |
| --- | --- | --- | --- | --- |
| USART2 TX | 7 | PG3 / AF7 | UART transmit to MangOH RX | OUT |
| USART2 RX | 9 | PF6 / AF7 | UART receive from MangOH TX | IN |
| GND | 11 | — | Common ground | — |
| +3V3 (opt) | 13 | — | Logic-level reference | — |

TX is PG3, not PF7. Earlier revisions of this table said PF7; the firmware has always driven PG3 (n6cam_io.h: USART2_TX_PORT GPIOG / GPIO_PIN_3). Confirm which J503 pin PG3 lands on against the schematic before probing — a cable wired from the old table puts the TX wire on a pin USART2 never drives.

All header logic is 3.3 V. Match this on the MangOH side — do not drive 5 V into PF6.

### 1.1.1 MangOH Yellow side (CN805 expansion connector)

UART1 is on CN805 pins 7–8 at 3.3 V, reaching the WP76 (1.8 V) through a FXMA108 auto-direction level translator. The Linux node is /dev/ttyHS0 (not ttyHSL1 — that node exists but isn’t muxed out to CN805).

Two things this part imposes:

- No pull-ups or pull-downs on its data lines. The datasheet is explicit, and the auto-direction one-shot misreads an externally driven edge as the far side taking over. Any resistor must be >50 kΩ, so the STM32’s ~40 kΩ internal pull-up is not acceptable — USART2’s pins are deliberately GPIO_NOPULL (n6cam_uart.c, _bsp_uart_init_gpio). USART1 keeps its default pull.

- Identify TX empirically, don’t trust a pinout. With the camera unplugged and sdvrApp running, whichever of pins 7/8 carries the +SDVRRDY burst on app restart sdvrApp is the WP76’s TX. That burst is a complete HDLC frame and is readable in any terminal at 115200 8N1: 7e 2b 53 44 56 52 52 44 59 3a 20 ... 0d 0a <crc-hi> <crc-lo> 7e

### 1.2 SD card

| Field | Value |
| --- | --- |
| Slot | Onboard microSD (J504) |
| Tested card | SanDisk Class 4 32 GB |
| Filesystem | FAT32 |
| Format | sd format CONFIRM (shell command, destructive) |
| Mount | sd query reports mounted after boot |

### 1.3 USB endpoints

| Endpoint | Path on Linux host | Purpose |
| --- | --- | --- |
| CDC ACM | /dev/serial/by-id/usb-STMicroelectronics_N6Cam_DEADBEEF-if02 | Operator shell, CDC self-update channel |
| UVC video | /dev/video2 (typical) | 800×600 H.264-encoded live preview from the kit |
| STLink VCP | /dev/serial/by-id/usb-STMicroelectronics_STLINK-V3_*-if01 | Trace UART (boot log + LINFO/LERROR output) |

The STLink VCP is the secondary read channel — the shell never echoes its trace there, but the firmware’s trace() calls land there.

## 2. Firmware architecture

The firmware is ThreadX-based with each subsystem in its own task. Producer/consumer wiring at the task boundary:

| Task | Priority | Owns | Talks to |
| --- | --- | --- | --- |
| camera_task | IMPORTANT | IMX335 + display pipeline | display, nn |
| display_task | IMPORTANT | H.264 encode + UVC TX | snapshot (every frame) |
| nn_task | CRITICAL | ATON inference + post-proc | snapshot, modem (on detect) |
| snapshot_task | DEFAULT | JPEG encode + dispatch | fx (SD), modem (UART upload) |
| fx_task | DEFAULT | FileX / SDIO | snapshot |
| modem_task | USER_INTERFACE | USART2 + HDLC framing | nn, shell, snapshot |
| shell_task | USER_INTERFACE | CDC ACM + lwshell | every other task via public APIs |
| registry_task | CRITICAL | NV-persistent config | shell (write), every task (read) |
| system_task | BACKGROUND | RTC, trace, FPS counters | trace UART |
| watchdog_task | CRITICAL | IWDG pet | — |

### 2.1 Detection → action pipeline

Camera                                  /dev/video2
                         │                                          ▲
                         │ frame                                    │
                         ▼                                          │
                    ┌────────┐    ┌────────┐    ┌──────────┐        │
                    │ camera │───►│   nn   │───►│ display  │────────┘
                    │  task  │    │  task  │    │   task   │
                    └────────┘    └───┬────┘    └──────────┘
                                      │ on object-count edge
                                      ▼
                              ┌───────────────────┐
                              │  snapshot_request │   ← shell `photo savesd|upload`
                              │       (atomic)    │     also flows here
                              └─────┬─────────────┘
                                    │ target=SD       target=UART
                                    ▼                 ▼
                            ┌───────────┐     ┌──────────────┐
                            │  fx_task  │     │  modem_task  │
                            │ FAT32 SD  │     │  USART2 HDLC │
                            └───────────┘     └──────────────┘

### 2.2 What the detector is given

Two DCMIPP pipes run off the one sensor, and they are not interchangeable:

| Pipe | Feeds | Size | Format |
| --- | --- | --- | --- |
| PIPE1 “main” | preview, UVC stream, overlay, uploaded photos | 800 x 600 | RGB565 |
| PIPE2 “ancillary” | the neural network, and nothing else | 256 x 256 | RGB888 |

Both are fed from the whole sensor. That is an invariant, not an incidental fact, and it is the one worth checking first whenever the overlay and the screen disagree: if the detector’s rectangle is smaller than the preview’s, there is picture the operator can see and the detector cannot, and no view of the boxes will ever say so. It was wrong once — the ancillary pipe took a 1944x1944 centre crop of a 2592x1944 sensor, so the left and right eighths of every scene were invisible to the network (ScopusQA #25).

frame grab prints both rectangles, and saves either picture:

python3 n6cam-grab-frame.py --source nn    -o nn_input.png     # the network's own input
python3 n6cam-grab-frame.py --source live  -o live_frame.png   # what the screen shows
# frame grab: sensor 2592x1944  nn-area 0,0 2592x1944  main-area 0,0 2592x1944
# frame grab: buffer 0x90030000, pipe filling 0x90000000

The second line is the other invariant. Both pipes are double-buffered, so a reader that asks the hardware which buffer it is using gets one that is half written — the top of the new frame spliced onto the bottom of the previous one. Invisible on a still scene, torn on anything moving. camera_get_buffer() hands out the buffer the DCMIPP has finished with; frame grab probe samples the status register if you ever need to confirm the pipe really is alternating.

The whole path, including where an injected frame joins it:

Detection path

## 3. Protocols

### 3.1 CDC shell protocol (M2 — already in production)

Text-line shell on the CDC ACM endpoint. Each command terminated with \n (or \r\n). Output lines end with \r\n. Echo controllable via echo on|off. The full surface is the Scopus SoW §4 table — see tests/run_tests.py for executable references. Highlights:

- detect start|stop|profile <det> <act>|profile query|simulate [N]

- notify enable|disable|trigger <code>|period <s>|query

- photo savesd|upload

- img size <H> <W>|quality 1..100|color YCBCR|RGB|CMYK|chroma 0|1|query

- rtc set DDMMYYYYHHMMSS|rtc

- irled on|off|query

- motion sense <0..100> <timeout_s>|query|read|selftest|simulate 0|1 (the board’s own movement, from the LSM6DSO32 IMU — not objects moving in the frame)

- sd query|ls|format CONFIRM

- frame upload|load|run|clear|query (NN test-frame injection)

- update app|model (CDC self-update, see §3.4)

- mdm <at-cmd>|test echo|test urc <line> (modem tunnel, see §3.2)

### 3.2 Modem channel (M3 — N6 side complete, awaiting MangOH)

Transport: USART2 @ 115200 8N1 with HDLC framing — 0x7E start/end flags, 0x7D escape with XOR 0x20, CRC-16/XMODEM appended big-endian. See vendor/.../Application/Core/Inc/hdlc.h for the C contract and tools/hdlc.py for the host-side mirror.

Wire shape for an AT command:

┌───────────────────────────────────┐
                          │ AT+CSQ\r\n  (4 bytes, big-endian   │
                          │            CRC, byte-stuffed)      │
                          └───────────────────────────────────┘
0x7E  41 54 2B 43 53 51 0D 0A  ━━ CRC-HI ━━ ━━ CRC-LO ━━  0x7E

If any payload or CRC byte is 0x7E or 0x7D, it’s split into 0x7D + (byte XOR 0x20).

Higher-level dispatch: each HDLC frame is one or more CRLF-terminated lines. The modem_task worker:

- Decodes the frame.

- If a synchronous caller is waiting in modem_send_at(), accumulates lines into the response buffer; on OK / ERROR / +CME ERROR: / +CMS ERROR: the wait fires.

- Otherwise (or for any +SDVR* line received outside a pending response), invokes the URC callback. shell_task installs a forwarder that re-emits the URC on the CDC shell stream so the operator sees +SDVRRDY:, +SDVRUPL:, etc. as they arrive.

Photo upload (SoW §8.2) uses the binary transport variant:

host → kit:  shell `photo upload`
kit  → modem: SDVR+SENDBIN=<ref>,"<tag>","<DDMMYYYYHHMMSS>",<ref>,<size>\r\n
kit  → modem: <size bytes of JPEG>                  (HDLC-framed, may span frames)
modem → kit: +SDVRUPL: START,"<tag>_<ref>"
modem → kit: +SDVRSRVR: OK                          (after HTTP POST succeeds)
modem → kit: +SDVRUPL: END,"<filename>"
modem → kit: OK

The whole SENDBIN + binary tail is serialised under modem_task’s TX mutex so a concurrent shell mdm <at> from the operator can’t interleave its frame between the prefix and the payload.

### 3.3 Server-bound notification (SoW §6)

When the kit fires a detect event (or any other notification-capable event), shell_task::_notify_emit() builds a JSON object:

{
  "ser":  4325412,
  "num":  10,
  "rsn":  16,
  "rsd":  3,
  "tim":  "20260523111111",
  "mtn":  0,
  "mod":  "",
  "bat":  0.0,
  "vol":  0.0
}

| Field | Meaning |
| --- | --- |
| ser | Kit’s MCU UID (HAL_GetUIDw0) |
| num | Per-boot rolling notification number |
| rsn | Reason bitmask: 1 NetReg, 2 MotStart, 4 MotStop, 8 Periodic, 16 People, 32 Vehicle, 64 PhotoEvent. MotStart/MotStop are the box being moved (inertial sensor); People/Vehicle are the scene |
| rsd | Reason-specific data (People: detected count; MotStart: the deviation that opened the episode, in mg; MotStop: how long it lasted, in seconds; PhotoEvent: 1=SD, 2=upload) |
| tim | RTC timestamp YYYYMMDDHHMMSS |
| mtn | Motion state of the unit when the notification was composed, read from the inertial sensor (1 while the box is being moved). Carried on every notification, not only on motion events |
| mod/bat/vol | Mode / battery / voltage placeholders |

Emitted on: - the CDC shell as +SDVRNTF: { … }\r\n - (when M3 wires up) over the modem channel via SDVR+NTFA=…

### 3.4 Safe-boot / bootloop guard

A bootloop counter lives in the flash-backed registry (boot_count field). registry_task_init bumps the counter and synchronously saves it before the NN, camera, or display tasks have a chance to start — so a crash anywhere downstream (bad model load, ATON HW init, camera bring-up, display init, NN inference) still gets counted on the next boot.

Once the counter crosses 3 consecutive boots without a healthy clear, shell_task enters safe mode:

- nn_task_detect_set(false) — NN auto-start suppressed (the typical bad-update failure mode is a bad model, so we stop running it).

- LERROR(TRACE_SHELL, "*** SAFE BOOT *** …") prints on the STLink VCP

  - CDC shell.

- Operator’s CDC shell stays alive; update app / update model work normally — kit recovers without SWD.

After ~60 s of healthy uptime (HEALTHY_BOOT_TICKS = 300 × 200 ms in the shell main loop), the counter is reset to 0 and the next reboot starts over.

Shell command:

> safeboot status         # current counter, threshold, safe-mode flag
boot_count = 1/3, safe_mode = no
safeboot status ok

> safeboot clear          # zero the counter (post-recovery)
> safeboot test           # set counter to 2 — next reboot engages safe mode
                          # (drill / verification aid)

Field recovery flow when a bad update app or update model has caused the kit to crash repeatedly:

- Kit reboots 3× in a row (registry counter goes 0 → 1 → 2 → 3).

- Boot #4: counter is already 3 → safe mode engages → NN doesn’t auto-start → kit is reachable on CDC.

- Operator pushes a known-good update app or update model.

- After the fix is in, safeboot clear to zero the counter.

- Reboot or wait ~60 s — kit returns to normal operation.

No FSBL changes, no SWD required, no boot-switch toggle.

### 3.5 Self-update protocol (M1 / A4)

See M1_M2_WORKFLOW.md for the diagram. Wire format:

host → kit:  update {app|model}\n
                         (1 line, plain text shell command)
host → kit:  UPDT
host → kit:  <size_le_4>
host → kit:  <crc32_le_4>
host → kit:  <size bytes of payload>
kit  → host: …Writing N bytes…
kit  → host: Done. Resetting...
             (and NVIC_SystemReset)

size cap: 1 MiB for app, 16 MiB for model. CRC must match (zlib crc32). Tool: ./n6cam-update.py --target {app|model} <file>.

## 4. Storage layout

### 4.1 xSPI NOR (MX66UW1G45G, 32 MiB)

| Slot | Address | Cap | Signed? | Self-update? |
| --- | --- | --- | --- | --- |
| FSBL | 0x70000000 | 108 KB today | yes (-hv 2.3) | no — SWD only |
| Application | 0x70400000 | ≤ 1 MiB | yes (-hv 2.3) | yes, via CDC update app |
| Model weights | 0x70600000 | ≤ 16 MiB | raw bytes (mmaped by ATON runtime) | yes, via CDC update model |

### 4.2 SD card layout

/Snapshot{N}.jpeg                            ← auto-snapshot on NN detect
/{serial}_{DDMMYYYY}_{HHMMSS}.rdy            ← `photo savesd` (SoW §7)
/{serial}_{DDMMYYYY}_{HHMMSS}.upd            ← `.rdy` renamed after a
                                                successful HTTPS upload
                                                (SoW §8.1; M3 wiring)

sd ls (chunked output via the shell — previously truncated past 38 entries, fixed in v1.3.2) shows everything in the FAT root.

## 5. Performance characteristics

| Metric | Value | Source |
| --- | --- | --- |
| NN inference (P50 / P95) | 25.0 ms / 25.5 ms | tests/run_tests.py, 5-iter soak (75 NN samples) |
| Camera FPS | 29 Hz | system_task FPS counter |
| App boot time (cold) | ~5 s | uptime poll after n6cam-update.py --target app |
| Multi-class model size | 3.4 MB INT8 (192×192, 80-class YOLOv8n) | tools/qat_artifacts/yolov8n_relu30_u8in_f32out.tflite |
| Compiled NPU blob size | 3.0 MB | vendor/.../Firmware/Model/network_atonbuf.xSPI2.raw |
| Detection accuracy (test set) | 55 / 56 PASS, 1 SKIP | results/test-report-20260523_165724.{html,pdf} |

## 6. Bench acceptance (Proposal v4 / A5)

### 6.1 Quick smoke (~1 min)

./modular-tools.sh test
# or, plain Python:
python3 tests/run_tests.py

Output: - results/test-report-<ts>.html — self-contained, with detection thumbnails inlined as base64 data URIs - results/test-report-<ts>.pdf — same content via headless Chrome - Stdout: PASS/FAIL summary table

Reference baseline: 55 PASS / 0 FAIL / 1 SKIP, NN P95 25.5 ms.

### 6.2 ≥ 24 h soak (A5 deliverable)

python3 tools/bench_soak.py --hours 24

What it does:

- Loops the full regression suite (~65 s/iteration → ~1330 iterations / 24 h).

- Tracks kit uptime between iterations — flags any drop as an unexpected reboot.

- Logs to results/soak-<ts>/soak.csv — per-iteration row: wall_s, kit_uptime_s, rebooted, pass, fail, skip, total, test_time_s, nn_med_ms, nn_p95_ms.

- Each iteration also drops a same-numbered HTML report under results/soak-<ts>/soak-NNNNN.html.

- Prints a summary at end: total iterations, pass rate, unexpected reboots, NN median + P95 across all samples.

Pass criterion: unexpected_reboots == 0 and fail == 0 over the full window. Verified 6-min smoke (97.5 % pass rate, 0 reboots, NN P95 25.5 ms — only failures are the documented T07.4/T11.6 known issues on cold-start).

### 6.3 M3 cellular plane validation (when MangOH lands)

- Wire J503 pin 7 (N6 TX) → MangOH UART RX, pin 9 (N6 RX) → MangOH UART TX, pin 11 → MangOH GND. Match 3.3 V logic levels.

- On the kit’s CDC shell:

- > mdm test echo            # HDLC framing roundtrip — verified locally
> mdm AT                   # should answer "OK" from the MangOH
> mdm AT+CSQ               # signal strength
> mdm SDVR+HOSTIP=…        # configure server
> mdm SDVR+PING=42         # echoes back "+SDVRPING: 42"
> photo upload             # captures + ships JPEG via SoW §8.2

- For pre-MangOH testing, use the host-side stub:

- sudo python3 tools/fake_modem.py /dev/ttyUSB0

- wired through any 3.3 V USB-UART adapter into J503. It answers AT/SDVR+ commands and consumes SDVR+SENDBIN binary tails so the full transport stack can be wrung out without the actual modem on the bench.

## 7. Known limitations

| ID | Symptom | Workaround |
| --- | --- | --- |
| T11.6 / crowd_13.jpg | Specific dense-crowd image triggers ATON hardfault; kit IWDG-resets in ~1 s, App boots clean | None — production live-mode sees a ~5-s glitch on this exact input then full recovery. Documented in MULTICLASS_STATUS.md. |
| 5_people.jpg under-count | Model reports 1-2 people instead of 5 | Model-quality limitation at 192×192 input. Closes with bigger input + longer training (Lane C of the upcoming improvement plan). |
| 1-class baseline kept | vendor/.../Model/_backup_person_only/ holds the original 17 ms/frame person-only model | Swap if a faster single-class baseline is needed; the rest of the App is class-count-agnostic. |

## 8. Repository layout

.
├── M1_M2_COVERAGE.md           — WBS coverage doc (v1.4.0)
├── M1_M2_WORKFLOW.md           — pipeline + self-update flow (v1.4.1)
├── MULTICLASS_STATUS.md        — model accuracy / stability journal
├── INTEGRATION_DOC.md          — this file (A6)
├── Scopus_SoW_v3.pdf           — authoritative SoW
│
├── docs/
│   ├── diagrams/               — graphviz sources + rendered PNGs
│   └── docx/                   — DOCX exports of the .md docs
├── results/                    — bench-test HTML + PDF + soak CSVs
├── captures/                   — live-detection JPEGs from the kit UVC
│
├── tests/
│   ├── run_tests.py            — main regression suite
│   ├── images/                 — deterministic test scenes
│   ├── c/test_hdlc.c           — HDLC host-test binary
│   └── test_hdlc_crosscheck.py — Python ↔ C HDLC equivalence check
│
├── tools/
│   ├── hdlc.py                 — Python HDLC encoder/decoder
│   ├── fake_modem.py           — host stub for M3 pre-MangOH bring-up
│   ├── bench_soak.py           — A5 ≥ 24 h soak script
│   └── qat_artifacts/          — saved model + compiled NPU blob
│
├── n6cam-update.py             — CDC self-updater (App / Model)
├── modular-tools.sh            — vendor-style build/sign/flash wrapper
│
└── vendor/n6cam.core.bsp/      — vendored SIANA BSP with our overlays
    └── Firmware/Application/Core/
        ├── Inc/{,Tasks/}*.h
        └── Src/{,Tasks/}*.c   — modem_task.c, hdlc.c, snapshot_task.c,
                                  shell_task.c are our additions/edits

## 9. Tags on master

| Tag | Scope |
| --- | --- |
| v1.0.0 | Baseline (single-class person detector working) |
| v1.1.0-multiclass | Multi-class person+vehicle detection plumbing |
| v1.2.0-multiclass-counts | NMS + bbox sigmoid fixed, counts match GT on clean cases |
| v1.3.0-accurate-bboxes | Dequantize zero-point bias fix — person@0.91 instead of 0.34 |
| v1.3.1-suite-green | 55 PASS / 0 FAIL / 2 SKIP, full bench acceptance |
| v1.3.2-sd-ls-fix | sd ls truncation fixed, photo savesd test passes |
| v1.4.0-m1-m2-delivery | M1 + M2 coverage doc + acceptance email |
| v1.4.1-m1-m2-docs | Workflow doc + diagrams + DOCX exports |
| v1.4.2-self-contained-report | HTML report embeds base64 images; auto PDF render |
| v1.5.0-m3-cellular-plane | M3 part 1: HDLC + mdm + SoW §8.2 photo upload (this doc) |

git log --oneline | head -20 for individual commits.

This document is the source. docs/docx/INTEGRATION_DOC.docx is generated from it and is regenerated, not edited:

pandoc INTEGRATION_DOC.md -o docs/docx/INTEGRATION_DOC.docx --resource-path=.:docs