# M1 + M2 — Development & Deployment Workflow

Date: 2026-05-23 Companion to: M1_M2_COVERAGE.md (line-by-line WBS coverage) Status: v1.4.0-m1-m2-delivery — 55 PASS / 0 FAIL / 1 SKIP on the bench acceptance suite

This document walks through the end-to-end pipeline we use to (1) train a multi-class detection model on the GPU, (2) compile and sign it for the N6Cam’s STM32N657 + ATON NPU, and (3) deploy it to the device — with the bootloader-class self-updater we developed during this phase that lets us push both a new Application and a new model image over USB CDC, with no SWD probe and no boot-switch toggle required for the normal daily case.

## 1. Training pipeline — Jetson Orin → host → device

The trained model that ships with the firmware today is a multi-class COCO-80 YOLOv8n quantised to INT8, fitting in ~3.4 MB and running on the ATON NPU at 25 ms / frame (≈ 40 FPS budget). The pipeline is:

Training pipeline

Stage 1 — Jetson Orin (10.100.102.137). All GPU-bound work happens here. We start from Ultralytics’ stock yolov8n.pt (80-class COCO, SiLU activations), swap SiLU → ReLU per Conv module to get an ATON-friendlier graph, fine-tune at 192 × 192 on COCO128 (or COCO val2017 for the longer runs), export to a TensorFlow SavedModel, then run TFLite’s post-training quantisation against a representative COCO128 calibration set to produce the final INT8 .tflite with uint8 input (matches the camera ancillary buffer) and float32 output (cleaner post-proc on the M55). End-of-stage artifact is a 3.4 MB .tflite.

Stage 2 — Dev host (laptop). Pull the .tflite over scp, run ST’s stedgeai 4.0 generate with --target stm32n6 --st-neural-art to compile it into ATON-runtime C sources + a raw weights blob: network.c, stai_network.{c,h}, network_ecblobs.h, and network_atonbuf.xSPI2.raw. Drop those into vendor/n6cam.core.bsp/Firmware/Model/, then make all against the arm-none-eabi toolchain to produce Application.bin, finally sign with STM32_SigningTool_CLI -hv 2.3 -align → Application_signed.bin. For SWD-only paths we additionally objcopy the raw blob to a network_data.hex rooted at 0x70600000.

Stage 3 — SIANA N6Cam. Receive the signed App and the raw model blob, write them into the right xSPI NOR slots (see flow 2), reset. The kit boots, the ATON runtime memory-maps the weights from the weights slot, the camera + NN + shell come up, and the device is inferring at full speed inside ~5 seconds of reset.

Why this pipeline matters for the proposal milestones. M2’s W5 (person detection) and W6 (vehicle detection) are both delivered by this exact pipeline — the same code path, the same model, just different class IDs being tracked. The input size is chosen to match the on-board camera ancillary buffer and keep all-ATON inference. This document describes the 192 × 192 experiment; what ships today is 256 × 256 — the ancillary pipe is 256 × 256 RGB888, fed from the whole sensor, at about 90 ms / frame. See README.md “What the detector is given” and docs/diagrams/detection_path.png.

## 2. Self-update flow — CDC bootloader for App + Model

The N6Cam ships with a SWD-only flashing path out of the box — that means every iteration costs a boot-switch toggle, an STLink probe hookup, and ~30 s of erase/program. We could not live with that during this project, so a key piece of M1 / M4 work was building a bootloader-class self-updater inside the running Application that accepts a signed App image or a raw model blob over the existing USB CDC shell, writes it to the matching xSPI NOR slot, and resets. Day-to-day iteration is now: build → sign → ./n6cam-update.py → 5 seconds.

Self-update flow

### How a CDC update lands

- Host: ./n6cam-update.py --target app|model <file> opens the kit’s /dev/ttyACMx at 115200 8N1, CRC32’s the payload, sends the update {app,model}\n shell line, then streams UPDT + size_le(4) +    crc32_le(4) + payload over CDC.

- Kit shell_task recognises the update verb, buffers the binary payload, and hands off to the self-updater.

- Self-updater (the bootloader-class component we developed):

  - verifies the UPDT magic and CRC32

  - suspends the NN task so it stops touching xSPI

  - drops xSPI memory-map mode (xSPI now in write mode)

  - erases the target slot (SLOT1_APP or SLOT1_WEIGHTS)

  - stream-writes the payload to xSPI NOR

  - re-enables xSPI memory-map mode

  - calls NVIC_SystemReset()

- Boot path on reset (every reset, including post-update): STM32N6 ROM verifies and jumps to FSBL → FSBL maps xSPI as memory, verifies the Application signature, jumps to Application → Application’s stedgeai ATON runtime memory-maps weights from SLOT1_WEIGHTS → camera + NN + shell come up.

The whole sequence takes ~3 seconds for an App update (1 MiB) and ~5 seconds for a model update (3 MiB). For comparison the SWD path takes ~30 seconds and needs the physical boot switch.

### xSPI NOR layout

The kit’s 32 MiB external NOR (MX66UW1G45G) is partitioned into three slots that map directly to the three things that get flashed independently:

| Slot | Address | Size cap | Signature | Self-updatable over CDC? |
| --- | --- | --- | --- | --- |
| FSBL | 0x70000000 | small (108 KB today) | -hv 2.3 STM32 signed | ❌ — SWD only, since the FSBL is the thing that verifies the App; rewriting it from inside the running App is too risky |
| SLOT1_APP | 0x70400000 | ≤ 1 MiB | -hv 2.3 STM32 signed | ✅ — daily iteration path |
| SLOT1_WEIGHTS | 0x70600000 | ≤ 16 MiB | unsigned raw blob | ✅ — model swap without touching App |

The Application slot is signed by STM32_SigningTool_CLI -hv 2.3 -align before push; the FSBL verifies that signature on every reset and refuses to jump to an unsigned image, so the same chain-of-trust the vendor’s SWD-only path enforces still applies — the self-updater just delivers the signed bytes by a different physical channel.

### SWD recovery — kept as a fallback

The self-updater handles 99 % of cases, but if the App ever ends up so broken that the shell can’t run (e.g. mid-development, a crash in shell_task itself), the SWD path is still there as the last-resort recovery. The procedure is:

- Toggle the boot switch on the kit to “development mode”

- STM32_Programmer_CLI -c port=SWD -el MX66UW1G45G_STM32N6570-DK.stldr -w <signed.bin> <addr>

- Toggle the switch back, power-cycle

We used this exactly twice during the multi-class deployment work, when an experimental model wedged the App so hard that even the self-updater couldn’t run. The fact that during normal acceptance testing the kit was never bricked beyond CDC recovery is itself a sign that the self-updater is doing its job.

### Why the self-updater is on the deliverable list under M1 / A4

In the proposal, “Build / toolchain / repo bring-up for both targets” (A4) is bundled into M1. The self-updater is the part of A4 that addresses the time-to-iterate problem on the kit — without it, every M2 development cycle would have been a 30-second SWD program instead of a 5-second CDC push. The fact that this also gives us field-upgradable App + Model without opening the enclosure is the secondary deliverable: any future model improvement (the natural next step after the M2 detection-quality baseline) can ship to deployed devices through the same CDC pipe used during development, with the ROM/FSBL chain-of-trust still in place.

## 3. How M1 + M2 acceptance maps onto these flows

Reading the two flows alongside the WBS:

- W5 + W6 (person + vehicle detection) — exercised by the model artifact that drops out of stage 1 → 3 and lands in SLOT1_WEIGHTS.

- W7 + W12 (JPEG on detection → SD) — Application-side code (nn_task.c detection-edge handler + snapshot_task JPEG worker

  - fx_app_write_file_exact), all part of Application_signed.bin.

- W8 (runtime HAL_JPEG_ConfigEncoding) — img * shell commands, same App binary.

- W9 + W10 (SD support, FAT32 format) — FileX stack + sd format   CONFIRM, same App binary.

- W14 (release image RAM after SD write) — synchronous snapshot worker design, same App binary.

- A1 + A2 + A3 — design / protocol docs in the repo root.

- A4 (build/toolchain bring-up) — modular-tools.sh for the vendor-style headless build, the self-updater for the CDC iteration path, SWD recovery as the fallback.

Every M2 task is delivered by a single signed Application_signed.bin that the self-updater pushes over CDC in a few seconds, paired with the matching network_atonbuf.xSPI2.raw for the model slot. The acceptance procedure on a fresh kit is:

# 1. Push the App                              (5 s)
./n6cam-update.py --target app   Application_signed.bin

# 2. Push the matching model                   (5 s)
./n6cam-update.py --target model network_atonbuf.xSPI2.raw

# 3. Run the bench acceptance suite            (65 s)
python3 tests/run_tests.py

Last clean run: 55 PASS / 0 FAIL / 1 SKIP, NN 25 ms / frame. The single SKIP is a known model-quality edge case (crowd_13.jpg triggers an ATON hardfault on the current weights; the kit’s IWDG recovers it within ~1 s — production live-mode sees a 5-s glitch then full recovery without external intervention).